ECS 17: Data, Logic, and Computing

Midterm 1

February 4, 2026

Notes:

- The midterm is open book, open notes.

- You have 50 minutes, no more: I will strictly enforce this.

- The midterm is graded over 70 points

- Please write your name at the top right of each page you turn in!

- Please, check your work! Also, do show your work

Part I (6 questions, each 5 points; total 30 points)

(These questions are multiple choices; in each case, find the most plausible answer)

- How much space would you need to store a 5 min song that has been sampled at 44.1 kHz, with each data point stored on 24 bits, in stereo (i.e. with two microphones)? Assume no compression.

  - About 80 Gbytes

  - About 80 Mbytes

  - About 106 Mbytes

  - About 800 Kbytes

- Let X be the number with the hexadecimal representation #AA and Y the number whose hexadecimal representation is #9D; which of these numbers T (in hexadecimal form) satisfies X-T=Y?

  - A

  - B

  - C

  - D

- Which of these bytes represents the letter P (uppercase) based on the ASCII code?

  - 01010001

  - 10100001

  - 01010010

  - 10100010

- A cooling fan in an industrial machine rotates at a maximum speed of 1200 revolutions per minute (rpm). You need to install a vibration sensor to monitor the fan's rotation frequency. Which of the following sampling rates is the most appropriate to monitor correctly the fan?

      - 20 Hz,

      - 30 Hz,

      - 40 Hz,

      - 50 Hz.

- We have a word with 2 letters. If we sum the ASCII representations of these 2 letters, we get #96. If, instead, the subtract the ASCII representation of the first letter from the ASCII representation of the second letter, we get #8. What is the word?

      - GO

      - OG

      - go

      - og

- Which binary number comes right after the binary number 101111?

  - 101112

  - 111111

  - 101110

  - 110000

Part II (two problems, each 10 points; total 20 points)

1) Complete the logic table corresponding to the logic gate shown below. Convert it into a Boolean expression (10 points)

| A | B | C | D | O |
| --- | --- | --- | --- | --- |
| 1 | 1 |  |  |  |
| 1 | 0 |  |  |  |
| 0 | 1 |  |  |  |
| 0 | 0 |  |  |  |

2) An engineer hands you a piece of paper with the following Boolean expression on it, and tells you to build a gate circuit to perform that function:

A⋅B+A⋅B

Draw a logic gate circuit for this function. Represent its logic table. Can you find a simpler logic gate that would perform the same operation on A and B? (10 points)

Part III (two problems, each 10 points; total 20 points)

1) You encounter a problem on an exam with three possible answers:

- Option 1

- Option 1 or Option 2, or both

- Option 2 or Option 3, or both

Unfortunately, you do not know what those options are, as the question has been omitted, but you know that only one answer (a, b, or c) is correct. Can you find that answer? Which of the 3 options was in fact correct? Explain your reasoning. (10 points)

2) On twilight island, there are two types of residents: day-knights and knaves. Day-knights tell the truth during the day and lie at night, while knaves always lie. At a time when you don’t know if it is day or night, you encounter Alice and Bob. Each makes a statement:

- Alice says: Bob is a day-knight.

- Bob says: Alice is a knave.

Is it day or night now? What are Alice and Bob? (10 points)

Appendix A: ASCII table